UNIVERSITY EXAMINATIONS

EXAMINATION FOR MAY/AUGUST 2024/2025 FOR BACHELOR OF SCIENCE IN COMPUTER SCIENCE

RCS 101  DIGITAL LOGICS.

DATE   06 / 08 /2025.		 TIME: 2 HOURS

GENERAL INSTRUCTIONS:

Students are NOT permitted to write on the examination paper during examination time.

This is a closed book examination. Text book/Reference books/notes are not permitted.

SPECIAL INSTRUCTIONS:

This examination paper consists Questions in Section A followed by section B.

Answer Question 1 and any Other Two questions.

QUESTIONS in ALL Sections should be answered in answer booklet(s).

- PLEASE start the answer to EACH question on a NEW PAGE.

- Keep your phone(s) switched off at the front of the examination room.

- Keep ALL bags and caps at the front of the examination room and DO NOT refer to ANY unauthorized material before or during the course of the examination.

- ALWAYS show your working.

- Marks indicated in parenthesis i.e. ( ) will be awarded for clear and logical answers.

- Write your REGISTRATION No. clearly on the answer booklet(s).

- For the Questions, write the number of the question on the answer booklet(s) in the order you answered them.

- DO NOT use your PHONE as a CALCULATOR.

- YOU are ONLY ALLOWED to leave the exam room 1 hour to the end of the Exam.

- DO NOT write on the QUESTION PAPER. Use the back of your BOOKLET for any calculations or rough work.

- Calculator will be required.

SECTION A (COMPULSORY)

Question (1) - (30Marks)

- Define the following terms as used in digital systems?   	 		(6 Marks)

- Bits.

- Bytes.

- Encoder.

- Decoders.

- Multiplexers.

- De-Multiplexers.

- Answer the following Questions, by selecting the correct choice, in the following multiple-choice questions.							(6 Marks)

- Which of the following gates, is a Universal gate?

| (A) NOT | (B) AND | (C) X–OR | (D) NAND |
| --- | --- | --- | --- |

- How many JK - Flip-Flops are required for mod–64 counter?

| (A) 5 | (B) 6 | (C) 8 | (D) 4 |
| --- | --- | --- | --- |

- What is the binary equivalent of the decimal number 368dec?

| (A) 101110000 | (B) 110110000 |
| --- | --- |
| (C) 111010000 | (D) 111100000 |

- In which of the following gates, the output is 1, if at least one input is 1

| (A) NOT | (B) AND | (C) OR | (D) NAND |
| --- | --- | --- | --- |

- The time required for a gate to change output state due to change in any inputs is referred to as?

| (A) Fall time. | (B) Decay time. |
| --- | --- |
| (C) Propagation time. | (D) Rise time. |

- Which of the following gates can be used in detecting Parity bits in error correction.

| (A) OR gate | (B) AND gate | (C) NOR gate | (D) XOR gate |
| --- | --- | --- | --- |

- Evaluate the following calculations. Show all the working.	      		     (10 Marks)

- DCBAhex				 - Convert to Octal.

- 17dec – 10dec   			 - Using twos complements.

- 11111111bin 		 	 - Convert to Gray code.

- 10011011bin + 10110101bin	 - Express the results in Decimal.

- FEDhex + CBAhex	 	 	 - Express the results in Hexadecimal.

- Derive the Boolean functions output of Y, with respect to the inputs A, B, C and D for the logic circuit below.		       					       	(4 Marks)

- Draw the logic	 Implement the Boolean expression below using suitable logic gates.											(4 Marks)

Yout = ((AB)C + (DE)(A̅BC) + A̅DE)

SECTION B (Answer Any Two Questions)

Question (2) - (15 Marks)

- Express the circuit below as a Boolean function.  		       		(6 Marks)

- Convert the following binary numbers, into Decimal Number.	      	(3 Marks)

10101010.1100112            Show all the working.

- Convert the following Decimal numbers, into Binary Number.	       	(3 Marks)

135.53110       		 Show all the working.

- Compare the Binary number codes and Gray code.		       		(3 Marks)

Question (3) - (15 Marks)

- Design a three bits Binary to Gray code converter, and then implement the logic circuit. You can use either Boolean simplification or the K-Mapping technique.	(6 Marks)

- Simplify the logic circuit below to its simplest form.		      		(3 Marks)

- Considering the logic circuit shown below, derive the Boolean function and hence a truth table, indicating all possible inputs (A, B and C) and specific output (Q).	(6 Marks)

Question (4) - (15 Marks)

- Considering the Boolean function shown below, draw an equivalent Logic circuit diagram.				 	       				(6 Marks)

Yo =A⨁BC+ B A⨁C +ABCD+ ADC

- Define the De-Morgan’s theorem.					       	(1 Marks)

- Using suitable truth tables, prove the De-Morgan’s theorem.     		(2 Marks)

- Design a Full adder from the first principle of digital circuit designing.	(6 Marks)

Question (5) - (15 Marks)

- Using Karnaugh Mapping technique of Boolean simplification, simplify the Boolean function below to the simplest term.  				       	(5 Marks)

YOut = ABCD+ABCD+ABCD+ABCD+ABCD+ABCD+ABCD+ABCD

- Considering the logic circuit shown below, Simplify the circuit, and then draw an equivalent Logic circuit diagram for the simplified circuits.			(5 Marks)

- Using any of the universal logic gates, implement the Exclusive NOR gates. 										 		(3 Marks)

- State any two types of ADC and DAC circuitry. 				(2 Marks)

END