UNIVERSITY EXAMINATIONS

EXAMINATION FOR JANUARY / APRIL 2024/2025 FOR BACHELOR OF SCIENCE IN COMPUTER SCIENCE

COURSE CODE:  RCS 108.                         COURSE TITLE:  COMPUTER ORGANIZATION                               & ASSEMBLY LANGUAGE.

DATE   -- / -- /2025	TIME: 2 HOURS

GENERAL INSTRUCTIONS:

Students are NOT permitted to write on the examination question paper during examination time.

This is a closed book examination. Text book/Reference books/notes are not permitted.

SPECIAL INSTRUCTIONS:

This examination paper consists Questions in Section A followed by section B.

Answer Question 1 and any Other Two questions.

QUESTIONS in ALL Sections should be answered in answer booklet(s).

# PLEASE start the answer to EACH question on a NEW PAGE.

# Keep your phone(s) switched off at the front of the examination room.

# Keep ALL bags and caps at the front of the examination room and DO NOT refer to ANY unauthorized material during the course of the examination.

# ALWAYS show your working.

# Marks indicated in parenthesis i.e. ( ) will be awarded for clear and logical answers.

# Write your REGISTRATION No. clearly on the answer booklet(s).

# For the Questions, write the number of the question on the answer booklet cover page in the order you answered them.

- DO NOT use your PHONE as a CALCULATOR.

- YOU are ONLY ALLOWED to leave the exam room 1 hour to the end of the Exam.

- DO NOT write on the QUESTION PAPER. Use the back of your BOOKLET for any calculations or rough work.

- Calculators may be required.

- Microprocessor Instruction sets will be provided.

SECTION A (COMPULSORY)

Question (1) - (30Marks)

- Describe the following terms as used in computer systems.	   		(7 Marks)

- I.S.A.

- Cache.

- B.I.O.S.

- P.O.S.T.

- Software.

- Firmware.

- Hardware.

- Describe FOUR major components of a microprocessor, stating their purposes.							                  			     	(4 Marks)

- Discus the purposes following components. 	     	     			(3 Marks)

    - Compiler.

    - Interpreter.

    - Assembler.

- A computer system has 18-bit control buses, 38 bits address buses, & 64 bits data buses.

- How much memory can the system handle?   	        	      	 	(2 Marks)

- What is the maximum size of word, the system can handle?  		(1 Marks)

- What will be the effect of executing the following intel 8085 instructions.	(4 Marks)

- XCHG

- STA 2500 H

- LHLD 2000 H

- LXI H, FFFF H

- Considering the CPUs and their Associated Addressing Modes.

- Explain what the term Addressing Mode means. 	     	 	(2 Marks)

- Describe FIVE Addressing modes used in intel’s 8085 Processors. 	(5 Marks)

- Differentiate between Parallel-Processing and Multi-Programming.  	(2 Marks)

SECTION B (Answer Any Two Questions)

Question (2) - (15 Marks).

- In details, discuss, the complex Fetch Execute Cycle process, as implemented during the instruction’s executions. 					 		(7 Marks)

- State FIVE categories / types of instructions based on their functionality.	(5 Marks)

- Name three major types of primary buses used by a computer system and hence describe their functionalities. 						 		(3 Marks)

Question (3) - (15 Marks).

- List THREE differences between the Harvard computer architectures and the Von Neumann computer architectures?	       			     		(3 Marks)

- State the purposes of the following I/O ports as used in computer. 		(4 Marks)

- Firewire Port.

- e-SATA Port.

- PS/2 Port.

- DVI Port.

- Assuming that the registers of Intel’s 8085 already have the data shown in the table below.

| Registers | Reg A | Reg B | Reg C | Reg D | Reg E | Reg H | Reg L |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Data | FFhex | 06hex | 04hex | ABhex | CDhex | F3hex | 8Ehex |

- What will be the data in the same register after executing the code / instructions below?									(4 Marks)

Instructions

XRA A

MVI B, F7 Hex

MOV D,A

MOV E,A

ADC B

INX D

XCHG

- Assemble the code above, and hence state amount of memory used. (4 Marks)

Question (4) - (15 Marks).

- Compare high-level programming languages and Low-level Programming.	     (2 Marks)

- State and explain FOUR major hardware related factors that have an impact, on computer performance.   							     (2 Marks)

- Outline FIVE major differences between R.I.S.C. and C.I.S.C as referenced in microprocessors.							    	     (5 Marks)

- State and Describe SIX major types of registers used found in a microprocessor (CPU). 		   						      	      		     (6 Marks)

Question (5) - (15 Marks).

- Describe all the flag register bits of the Intel 8085 microprocessor.		     (5 Marks)

- Write a program to multiply two numbers stored in the memory locations 2000hex & 2001hex respectfully, and then store the results from the operation into the memory location 2010hex.	       							     (6 Marks)

(Refer to the instruction set for Intel 8085 provided on page 5 and 6).

- Considering the assembly code below:

| ;<Program > | Comments: |
| --- | --- |
| JMP START |  |
| ; data |  |
| ; code |  |
| START: |  |
| MVI A, FFhex |  |
| LXI H, 20FFhex |  |
| Loop: |  |
| MOV M, A |  |
| DCR L |  |
| JNZ Loop |  |
| HLT |  |

- State what the code is used for.				     	(2 Marks)

- Comment on each statement to describe purpose.			(2 Marks)

(Refer to the instruction set for Intel 8085 provided on page 5 and 6).

Intel’s 8085 Microprocessors Instruction Set summary provided in the next two pages.

INTEL 8085 MICROPROCESSOR INSTRUCTION SET:

END